# Scaling to Module/Pack Level

The developed framework performs detailed cell-level analysis including data preprocessing, degradation modeling, anomaly detection, and RUL prediction using both statistical and machine learning approaches (including CEF-based modeling). To extend this framework to module and pack level, a scalable data-driven methodology is proposed.

### 1. Cell-to-Cell Variation

The current implementation operates at a single-cell level, where degradation trends, anomaly detection, and RUL prediction are computed using multiple models.

To scale this:

- The same pipeline (preprocessing → degradation modeling → anomaly detection → RUL estimation) is applied independently to each cell in the module/pack.

- Each cell generates its own:

  - Capacity degradation curve

  - CEF-based features

  - Anomaly labels (Isolation Forest, LOF, K-Means, Mahalanobis distance)

  - RUL predictions (polynomial extrapolation, ML, LSTM)

- Pack-level health is then derived by:

- Identifying the minimum RUL across all cells

- Detecting cells with early anomaly signatures

Thus, the framework scales naturally by treating the battery pack as a collection of parallel cell-level models, enabling weakest-cell identification.

### 2. Temperature Gradients

Although temperature data is not available in the dataset, hence it is not explicitly modeled in the current degradation or RUL prediction framework.

For scaling:

- Temperature can be included as an additional input feature in:

  - CEF-based feature set

  - Machine learning models (Linear Regression with SHAP analysis, LSTM)

- Since each cell in a pack may operate under different thermal conditions:

  - Each cell’s model can use its own temperature profile (If sensors are used enough)

- This allows:

  - Indirect capture of temperature impact on degradation

  - Improved RUL prediction under varying thermal conditions

This extension does not require modification of the existing pipeline, only feature augmentation.

### 3. Balancing Inefficiencies

In real battery packs, cells are connected in series and controlled using a BMS, leading to balancing inefficiencies and uneven cycling.

In the current framework:

- These effects are not explicitly modeled

- However, they are implicitly captured through:

  - Variations in capacity degradation trends

  - Differences in CEF values

  - Detection of anomalous cycles using unsupervised methods

Cells experiencing higher stress or imbalance naturally appear as:

- Faster degrading cells

- Outliers in clustering/anomaly detection

Thus, balancing inefficiencies are indirectly reflected through data-driven deviations and anomaly patterns.

### 4. Trade-off: Accuracy vs Computational Scalability

- Applying the full pipeline (including LSTM and ML models) to every cell increases computational complexity, especially for large battery packs.

- To address this:

  - Simpler models (polynomial extrapolation, linear regression) can be used for all cells

  - Advanced models (LSTM, SHAP-based ML) can be applied selectively to:

    - Cells with anomalies

    - Cells with low predicted RUL

This creates a balance between:

- High accuracy (ML + LSTM for selected cells)

- High scalability (simpler models for full pack)

### Conclusion

The proposed methodology can be extended to module and pack level since the Combined Efficiency Factor (CEF) is derived from Coulombic Efficiency (CE) and Energy Efficiency (EE), both of which can be computed using pack-level current and voltage measurements. As the current remains identical across series-connected cells, pack-level measurements are sufficient to estimate these efficiencies. However, it should be noted that pack-level CEF represents aggregated behavior and may not directly capture cell-level imbalances, thereby requiring complementary cell-level monitoring for precise diagnostics.